Dict keys for vo_create_box_model

Input dict:

- Main Params

  - Key: ImageToTrain

Type: RGB Image

Description:  Image used to create the shape model

  - Key:  BoxRegion

Type: Region

Description: Define the size of the box in pixels

  - Key: WorkRegion

Type: Region

Description: Define ROI where the boxes can be found.

  - Key: ShapeROI

Type: Region

Description:  Region where the image can be trained to obtain the shape model contour

  - Key:  BoxSize

Type: dict

Description: Real box size in millimetres

    - Key: Large

    - Key: Width

    - Key: High

- Optional Params

  - Key: Filters

Type: dict

Description:  Dict of all filters used to find the shape models in the best way.

    - Key: Channel

Type: Int

Description:  Select channel you want to use, number 4 is used for rgb_to_gray function

    - More filters …

  - Key: GetTopROI

Type: dict

Description:  Params to get top ROI

    - Key: MinArea

Type: int

Description:  Min area to consider a correct top region

    - Key: DeltaZ

Type: int

Description:  DeltaZ used to determine the steps needed to go down to find a complete layer

    - Key: Maxcompactness

Type: int

Description:  Max compactness of the region to consider it a correct top region

    - Key: MinRectangularity

Type: int

Description:  Min Rectangularity of the region to consider it a correct top region

    - Key: DilationRect

Type: int

Description:  Dilate final region

  - Key: SMParams

Type: dict

Description:  Params for shape model

  - Key: CheckCandidates

Type: dict

Description:  Params to determine if it is a correct candidate

    - Key: MinScoreCandidate

Type: int

Description: Once the possible candidates have been found, their Score value is checked, a loop is executed starting from the nominal score value to the “MinScoreCandidate”.

    - Key: MinScaleCandidate

Type: int

Description:

    - Key: ScoreCandidate

Type: int

Description:

    - Key: ScaleCandidate

Type: int

Description:

    - Key: MinScoreSM

Type: int

Description:

    - Key: PercentageBox

Type: int

Description:

  - Key: OrderCandidates

Type: string

Description:  Arrange the detected candidates

    - Key: OrderType

Type: string

Description:

  - Key: GetPose

Type: dict

Description:  Manipulate box pose

    - Key: ErosionCore

Type: int

Description:  Get core pose

    - Key: OffsetX

Type: int

Description:

    - Key: OffsetY

Type: int

Description:

    - Key: OffsetAngle

Type: int

Description:

  - Key: SecondRecipe

Type: dict

Description:  Second Recipe for multireference.

    - Key: Name

Type: String

Description: Name of second recipe

    - Key: Recipe

Type: HDict

Description: Second recipe

Output Dict

- Key: TrainDict

Type: dict

Description:  Dict where see all trained params

- Key: FiltersResult

Type: dict

Description:  Dict to see how the filters work

- Key: ModelID

Type: Handle

Description:  Shape Model ID

- Key: ModelContour

Type: Region

Description:  Region of trained shape model

- Key: StartAngle

Type: real

Description:  initial angle of box region in radian

- Key: SecondRecipe

Type: real

Description:  initial angle of box region in radian

- Key: Error

Type: Dict

Description:  Show all procedure errors

Dict keys for vo_find_multi_boxes

Input dict:

- Key: RecipeDict

Type: dict

Description:  Output dict of vo_create_box_model

- Key: InputImage

Type: Image RGB

Description:  Image where find candidates

- Key: DepthMap

Type: Depth map image

Description:  where obtain candidate position

- Key: RobotPose

Type: Dict

Description:

  - Key: ToolHCam (optional)

Type: Pose

Description: Calibration pose

  - Key: BaseHTool (optional)

Type: Pose

Description:  Real robot pose

  - Key: ZReinforce (optional)

Type: real

Description:  high of pallet from robot base position

- Key: RealHeight (optional)

Type: int

Description:

Output Dict

- Key: Candidates

Type: dict

Description:  Dict where see al train params

  - Key: [Num of candidate]

Type: dict

Description:  Dict of candidate

    - Key: Score

Type: int

Description: Score of candidates

    - Key: Contour

Type: Region

Description:  Contour of candidate

    - Key: BoxRegion

Type: Region

Description: Box Region

    - Key: Pose

Type: Tuple[X, Y, Z]

Description: Box pose (mm)

    - More information…

- Key: FiltersResult

Type: dict

Description:  Dict to see how the filters work

- Key: LayerRegion

Type: Region

Description:  Top layer region.

- Key: ImageResult

Type: Image RGB

Description:  Show all candidates detected

- Key: Error

Type: Dict

Description:  Show all procedure errors

  - Key: General

Type: String

Description:  Show general description of exception

  - Key: Exception

Type: Exception string

Description:  Show complete description of exception